Annex 14 – Site Report

- Since the last renewal & variation of the IPPC permit, infilling at the Ghallis landfill has continued until the point where it is estimated that landfill lifetime is expected to last till circa 2026 at current landfilling rates, given planned infrastructural and operational improvements. Operations at the landfill tipface are expected to continue as per current practice; however, pre-treatment of wastes to be landfilled is expected to improve given the planned initiatives to improve segregation of wastes at point of generation and subsequent collection.

- The environmental monitoring carried out to date has been submitted to the ERA via the regular submission of the Annual Environmental Report, as required by IPPC permit conditions. The following points are worthy of note:

- The air quality monitoring results have been generally satisfactory; this requirement has been dropped from the latest revision of the Environmental Monitoring Programme (refer to Section 6 in the EMP)

- For groundwater  exceedances in sodium and chloride and the resultant conductivity were only noted in the private boreholes. No exceedances were reported for the landfill boreholes.

- Odour monitoring results have indicated that odours have been noted at various points of the site perimeter however as per offsite sniff testing results  the propagation of the odour past the site boundary throughout the year was minimal. The use of daily cover is considered to have contributed to containing the impact.  Other  improvements consisted of  (1) deviating biodegradable and odorous wastes away from landfill (through improved segregation at point of source, and maintained in the subsequent collection system) and  anaerobic digestion at Malta North and (2) ensuring sufficient processing capacity to handle all the biodegradable wastes. Wasteserv invested and installed a misting system to mitigate odours.