Integrated IPPC Permit Renewal and Variation Application: IP 0002/05/Eii

Summary of the planned changes at Combino

The planned changes at Combino include:

- Additional level built as per PA/04360/15 to house quality control laboratories

- The use of a 40 feet container to store filters for HVACs or other non-hazardous material such as furniture

- Changing in Internal Layout at ground floor and first floor level

- New Container for storage of flammable material

- Introduction of new equipment in Production area (all in place): New compression machine, New Banding Machine and a New Coater.

- Four new emission points related to labs: PS7: PSD Enclosures extraction, PS8, PS 9 & PS10: Fume hoods in Chemistry Area 2nd Floor

- New emission points from Coater and Banding Machine (PS3h & PSHi) & HVAC PS11 a b c

Executive context

All machines are installed indoors in an enclosed room within the manufacturing facility of Combino Pharm, which itself is located in a designated industrial area. This location and installation context means that potential environmental impacts are contained and controlled. That greatly limits off-site noise/air effects. Residual impacts will therefore be via: (i) indoor process exhaust ducts (and any connected abatement), (ii) cleaning/CIP effluents, (iii) consumables and maintenance wastes, and (iv) electricity and utilities.

Planned Changes A, B, C

The additional level built as per PA/04360/15 to house quality control laboratories, and the changing of the internal layout at ground floor and first floor level have not changed any of the activities or processes carried out on the site. The changing rooms, showers and bathrooms have been extended and the lab space has been extended however the processes being carried out have remained the same.

The 40 foot container is a standard shipping container. It is placed in a designated area (see site plan). It is being used for the storage of filters for HVACs or other non hazardous material such as furniture. This is not expected to have any Environmental Impacts.

A prefabricated, fire-rated outdoor container used solely for storage of closed, labelled packages of flammable liquids (no mixing or dispensing inside). This material was previously stored in small quantities in the Medichem external Warehouse or inside laboratory areas. This is an improvement from an environmental point of view.

- Container for storage of flammable materials

A prefabricated, fire-rated outdoor container used solely for storage of closed, labelled packages of flammable liquids (no mixing or dispensing inside). Typical features assumed for best practice: weatherproof enclosure, passive or mechanical ventilation, impermeable floor and integral secondary containment (bund/sump), earthing points, and fire safety equipment. Location is within the site perimeter in an industrial area with controlled access.

Negligible point-source emissions are expected. Potential fugitive VOCs arise only when doors are opened and from any pressure-relief/vent grills. Items are stored in closed containers in this area, therefore, emissions are only considered a possibility in the case of a spill.

Summary of the container key specs:

- Designed for safe storage of flammable liquids in sealed containers (drums, cans, or IBCs).

- Sump (bund) capacity: Integral leak-tight collection sump (secondary containment) sized in line with standards (≥ 1000 L). Meets the 110% of largest container or 25% of total stored volume rule for secondary containment.

- Ventilation: Natural ventilation slots integrated into the container walls; optional mechanical ventilation system maintains safe vapour concentrations below occupational and explosion limits.

- Explosion / ATEX & electrical protection: Fitted with ATEX-compliant ventilation fan and electrical components. Container body and fittings designed for use in Zone 1/2 hazardous areas.

- Fire resistance: Fire-rated construction, tested to 90 minutes (REI 90) fire resistance. ATEX/Ex-rated components and ventilation are specified to avoid ignitable vapour-air mixtures. These features reduce likelihood of large-scale environmental releases due to fire/explosion

- Newly Introduced Equipment

Ei. Compression Machine

The below is a summary of potential environmental impacts associated with the operation of the Compression Machine installed at our pharmaceutical manufacturing facility.

1. Emissions to Air

- The machine itself does not generate gaseous emissions, combustion by-products, or volatile organic compounds (VOCs).

- The only potential airborne release is powder dust generated during compression. This is controlled by an integrated HEPA filtered dust extraction system that captures particulates at source and directs them to a dedicated dust collector. There is no specific emission point to atmosphere.

- The product is being handled entirely in a closed system, thus reducing possible emissions.

- The equipment is in a clean room with a system of HEPA filters in place. These are changed at regular intervals as per the organisation’s preventive maintenance programme, and other GMP requirements.

- No solvents, fuels or combustion systems are associated with the machine.

2. Waste Generation

- Solid waste: Mainly consists of collected product dust from the extraction system and rejected product from quality control. The integrated dust extraction and dedusting systems improve powder recovery, limiting product loss. These are classified as pharmaceutical process waste and handled in line with GMP and Maltese waste regulations.

- Oils and lubricants: The machine features an automatic oil lubrication system for tooling and cam tracks. Any oil that is collected from maintenance and disposed of as hazardous waste, in line with Maltese waste regulations.

3. Noise to the Environment

- Since the unit is installed in an enclosed room within a GMP facility, and the site is within a designated industrial area, no external environmental noise impact is expected, or has been observed during trials.

4. Resource Efficiency

Energy:

- The main drive motor is rated at 18.5 kVA. Additional power demand arises from auxiliary systems such as dust extraction, vacuum conveying, and cooling. While this will slightly increase site energy consumption, the impact is proportionate to normal pharmaceutical manufacturing operations.

- The machine incorporates a torque-drive motor with electronically adjustable speed, which optimises power use across production cycles.

Wastewater:

- The press operates with a closed-loop water cooling system for the torque drive and electrical cabinet. This prevents water discharge during operation.

5. Other Considerations

The machine complies with EU Machinery, ATEX, and EMC Directives, CE marking requirements, as well as GMP and FDA regulations.

It is fully enclosed within the plant, with separation of GMP (cleanroom), preventing environmental release.

Eii. Coating Machine

1. Emissions to Air

- See Gi below

2. Waste

- Wastewater: Coater generates CIP effluent containing coating residues (pigments, binders, possible solvent traces). This type of waste was already generated on site. These are classified as process waste and handled in line with GMP and directed to the waste water treatment plant. Waste from maintenance (oils, lubricants, etc) may also be generated from time to time. These are handled in line with local waste management legal requirements and disposed of via authorised providers.

3. Noise

- Noise is primarily generated from process air fans. It is contained within an enclosed manufacturing room; off-site impact negligible.

4. Resource & Energy Use

Efficiency features:

- The machine features shorter batch times, using less energy.

- A circulatory CIP system reduces water and detergent use.

- While the equipment will slightly increase site energy consumption, the impact is proportionate to normal pharmaceutical manufacturing operations.

5. Other Relevant Considerations

- Designed for high containment, reducing occupational and fugitive emissions.

Eiii. Banding Machine

1. Emissions to Air

- See Gii below

2. Waste

- Solid waste: Capsule shells, banding solution residues, panel and fan filters. This type of waste was already generated on site. Waste from maintenance (oils, lubricants, etc) may also be generated from time to time. All waste is handled in line with local waste management legal requirements and disposed of via authorised providers.

3. Noise

- Noise sources are primarily from the two fans for drying airflows. It is contained within an enclosed manufacturing room; off-site impact negligible.

4. Resource & Energy Use

- Energy use arises from two fans operating around the drying system. While this will slightly increase site energy consumption, the impact is proportionate to normal pharmaceutical manufacturing operations.

- Solvents will only be used in small quantities, keeping use of chemicals to a minimum.

5. Other Relevant Considerations

- The cleaning of this machine does not require any additional product or resource that is not currently in use in Combino.

- Machine is fitted with safety interlocks and alarms for vapour extraction in ATEX mode.

- Four new emission points related to labs (PS7 – 10)

Three new fume hoods and another extraction point have been added however the processes being carried out in the areas under extraction are the same as had previously been carried out. Monitoring on the previously existing points (PS5 and PS6) was carried out in 2022 and results provided to ERA. Please refer to attachments for results of monitoring carried out and map showing location of all point sources.

- New emission points from Coater and Banding Machine (PS3h & PS3i) & HVAC PS11 a b c

Gi. Coater PS3h

Coater equipped with active and static deduster. The latter has a HEPA filter installed. This is to filter any dust or spray being generated during the process. Prior to being emitted to atmosphere.

Gii. Banding Machine PS3i

The main emissions are: warm air with minor vapour traces. Exhaust is routed through its own AtEx extraction system.

Giii. HVAC PS11 a b c

New HVAC system installed to reduce the dependency on our main HVAC. Therefore, rooms which were previously on that main unit have now been reassigned to this new HVAC. It follows the same abatement systems as the previously installed systems (ie: prefilter in room to prevent escapes to atmosphere). It has 2 secondary emission points (b and c) to atmosphere which are only used when processes are being carried out inside the banding and coating rooms. During the bulk of our manufacturing operations, b and c will not be emitting to atmosphere.

Overall Conclusion

The new laboratory floor and change in layout do not change any functions or increase the footprint of the building and thus do have a significant impact on the environment.

The newly introduced machines do not present significant environmental risks beyond those typical of enclosed pharmaceutical production equipment. Overall, the equipment has a limited and well-managed environmental footprint within our facility.

The other newly introduced storage containers (flammables and non-hazardous items) also does not pose any risks to the Environment.

The emission points installed are the same as those which have previously been situated on site.

Therefore, it is not expected that the new equipment will pose a significant impact to the Environment.

# Application for an Environmental Permit –

# FORM B

# Materials and Equipment

  - Materials and Chemicals used on site

Are any new materials and/or chemicals (excluding fuels) intended to be used in any of the proposed activities?  Yes

If No, proceed to Section 4.3.

If Yes, list below all new materials (excluding fuels) used on site and provide details in the table below:

See table overleaf.

| Material Code | Name of Material/ Chemical | CAS Number/s | Maximum Storage Capacity on Site | Annual Consumption | Storage Arrangements | Containment and/or Measures |
| --- | --- | --- | --- | --- | --- | --- |
| M1 | Ethyl alcohol anhydrous | 64-17-5 | 20 litres | 200 litres | Original packaging stored in internal, temperature controlled warehouse. Placed on secondary containment due to liquid state. | Placed on secondary containment due to liquid state. Very low quantities stored at any given time. |
| M2 | Hypromellose (Pharmacoat 603) | 9004-65-3 | 20kg | 80Kg | Original packaging stored in internal, temperature controlled warehouse | None necessary over and above original packaging. |
| M4 | METHACRYLIC ACID COPOLYMER A (EUDRAGIT L100) | 25086-15-1 | 20kg | 200Kg | Original packaging stored in internal, temperature controlled warehouse | None necessary over and above original packaging. |
|  | METHACRYLIC ACID COPOLYMER B (EUDRAGIT S100) | 25086-15-1 | 20kg | 80Kg | Original packaging stored in internal, temperature controlled warehouse | None necessary over and above original packaging. |
| M5 | MICROTALC PHARMA 8 | 14807-96-6 | 20kg | 80kg | Original packaging stored in internal, temperature controlled warehouse | None necessary over and above original packaging. |
| M6 | DIBUTYL SEBACATE | 109-43-3 | 20kg | 60Kg | Original packaging stored in internal, temperature controlled warehouse. Placed on secondary containment due to liquid state. | Placed on secondary containment due to liquid state. |
| M7 | ISOPROPYL ALCOHOL | 67-63-0 | 200 litres | 3,000 litres | Original packaging stored in internal, temperature controlled warehouse. Placed on secondary containment due to liquid state. | Placed on secondary containment due to liquid state. Low quantities stored at any given time. |

Add more rows as required.

An MSDS sheet is provided for each material and/or chemical listed above.

  - Toxic Materials used on site

Do any of the materials listed in Section 4.1 above have any of the below listed Hazard Codes?  No

If No, proceed to Section 4.3.

If Yes, tick the appropriate Hazard Codes in the below table and provide the corresponding material code/s.

NOT APPLICABLE

| Hazard Codes | Tick | Material Code / s | Hazard Codes | Tick | Material Code / s |
| --- | --- | --- | --- | --- | --- |
| H310 | ☐ |  | H361d | ☐ |  |
| H318 | ☐ |  | H370 | ☐ |  |
| H330 | ☐ |  | H372 | ☐ |  |
| H331 | ☐ |  | H400 | ☐ |  |
| H332 | ☐ |  | H401 | ☐ |  |
| H340 | ☐ |  | H402 | ☐ |  |
| H341 | ☐ |  | H410 | ☐ |  |
| H350 | ☐ |  | H411 | ☐ |  |
| H351 | ☐ |  | H412 | ☐ |  |
| H360F | ☐ |  | H413 | ☐ |  |
| H360D | ☐ |  | H420 | ☐ |  |
| H361f | ☐ |  |  |  |  |

# Waste Management

  - Waste produced on site

List of new waste proposed to be produced on site and all information as requested below:

| Waste Code | Type of Waste | EWC Code |  |  |  |  |  |  | Maximum Amount Stored on site at any one time | Storage and Containment Arrangements | Permit Number for the facility accepting waste (IP/EP XXX/YY) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| WM1 | Non Halogenated solvents | 0 | 7 | 0 | 5 | 0 | 4 | * | 1000L | Stored in 1000Ltr IBCs on bunded pallets Outside/in a metal shed | EP 0025/21 |

Add more rows as required.

# Air and Noise Emissions

  - Point Emission Sources

Are any point source emissions to air proposed to be present on site?  Yes

Current PS1 – PS6 remain unchanged

If No, proceed to Section 7.3.

If Yes, provide the below information for all point source emissions:

| Emission Code | Source | Content of emission | Abatement Measures | Vent / Stack Height (m) |
| --- | --- | --- | --- | --- |
| PS7 | PSD Enclosures extraction | Total VOC as C | None | 0 |
| PS8 | Fume hood in Chemistry Area 2nd Floor | Total VOC as C, TSP | None | 0 |
| PS9 | Fume hood in Chemistry Area 2nd Floor | Total VOC as C, TSP | None | 0 |
| PS10 | Fume hood in Chemistry Area 2nd Floor | Total VOC as C, TSP | None | 0 |
| PS3h | Coater | Minor traces of diluted isopropanol | HEPA Filter | 2.5m |
| PS3i | Banding Machine | Minor traces of diluted ethanol | None | 2.5m |
| PS11a | HVAC Production Room (main exhaust) | Particulate matter, Total VOC as C | Pre filters in room | 0 |
| PS11 b & c | HVAC Production Room (secondary exhaust) | Particulate matter, Total VOC as C | Pre filters in room | 0 |

Add more rows as required.

  - Air Monitoring

Is any monitoring on the point source emissions currently being performed?  No

Monitoring on the existing points (PS5 & PS6) was carried out in 2022 and results provided to ERA. The processes carried out in these newly added points are the same as those carried out in the existing points.

If No, proceed to Section 7.3.

Emissions to Air Site Plan

11a b c

3e 3f 3g 3h 3i

PS1 Boiler

PS2 Generator

PS3a HVAC production process room

PS3b HVAC production room

PS3c HVAC production room

PS3d HVAC packaging room

PS3e Fluid bed drier & coater

PS3f Fluid bed drier emergency stack

PS3g Coater

PS3h Coater

PS3i Banding Machine

PS4 Fumehood vent

PS5 Fumehood vent

PS6 Fumehood vent

PS7 PSD Enclosures Extraction

PS8 Fumehood vent

PS9 Fumehood vent

PS10 Fumehood vent

PS11a b c HVAC production room

PS1 Boiler

PS2 Generator

PS3a HVAC production process room

PS3b HVAC production room

PS3c HVAC production room

PS3d HVAC packaging room

PS3e Fluid bed drier & coater

PS3f Fluid bed drier emergency stack

PS3g Coater

PS3h Coater

PS3i Banding Machine

PS4 Fumehood vent

PS5 Fumehood vent

PS6 Fumehood vent

PS7 PSD Enclosures Extraction

PS8 Fumehood vent

PS9 Fumehood vent

PS10 Fumehood vent

PS11a b c HVAC production room

8  9  10

QC Laboratories

QC Laboratories

7

7

6

6

5

5

4

4

3b

3b

Production

Production

Utilities

Utilities

3c

3c

3a

3a

3d

3d

2

2

1

1

Location of new equipment